/*

* ESP32-C6 Ultimate Clock System (English Version)

* Hardware: ESP32-C6 WROOM1 + DS3231 + 0.96" OLED(128x64)

* Date: 2025-10-21

*/

#include <Wire.h>

#include <RTClib.h>

#include <Adafruit_GFX.h>

#include <Adafruit_SSD1306.h>

#include <WiFi.h>

#include <time.h>

// ==================== Configuration ====================

// WiFi Settings

const char* ssid = "XIUYUAN";           // Change to your WiFi name

const char* password = "92511111";   // Change to your WiFi password

// NTP Server Configuration

const char* ntpServer1 = "ntp.aliyun.com";

const char* ntpServer2 = "ntp1.aliyun.com";

const char* ntpServer3 = "cn.ntp.org.cn";

const long gmtOffset_sec = 8 * 3600;        // UTC+8 Beijing Time

const int daylightOffset_sec = 0;

// I2C Pins

#define I2C_SDA 6

#define I2C_SCL 7

// OLED Configuration

#define SCREEN_WIDTH 128

#define SCREEN_HEIGHT 64

#define OLED_RESET -1

#define OLED_ADDR 0x3C

// Page Switch Interval (seconds)

#define PAGE_SWITCH_INTERVAL 5

// ==================== Global Objects ====================

RTC_DS3231 rtc;

Adafruit_SSD1306 display(SCREEN_WIDTH, SCREEN_HEIGHT, &Wire, OLED_RESET);

// Weekday Names

const char* weekDays[] = {"Sun", "Mon", "Tue", "Wed", "Thu", "Fri", "Sat"};

// Page Status

int currentPage = 0;

unsigned long lastPageSwitch = 0;

// ==================== Lunar Calendar Data ====================

// Lunar data table (2000-2050)

const unsigned int lunarInfo[] = {

0x04bd8, 0x04ae0, 0x0a570, 0x054d5, 0x0d260, 0x0d950, 0x16554, 0x056a0, 0x09ad0, 0x055d2,

0x04ae0, 0x0a5b6, 0x0a4d0, 0x0d250, 0x1d255, 0x0b540, 0x0d6a0, 0x0ada2, 0x095b0, 0x14977,

0x04970, 0x0a4b0, 0x0b4b5, 0x06a50, 0x06d40, 0x1ab54, 0x02b60, 0x09570, 0x052f2, 0x04970,

0x06566, 0x0d4a0, 0x0ea50, 0x06e95, 0x05ad0, 0x02b60, 0x186e3, 0x092e0, 0x1c8d7, 0x0c950,

0x0d4a0, 0x1d8a6, 0x0b550, 0x056a0, 0x1a5b4, 0x025d0, 0x092d0, 0x0d2b2, 0x0a950, 0x0b557

};

const char* lunarMonths[] = {"Jan", "Feb", "Mar", "Apr", "May", "Jun",

"Jul", "Aug", "Sep", "Oct", "Nov", "Dec"};

// Lunar Date Structure

struct LunarDate {

int year;

int month;

int day;

bool isLeap;

};

// ==================== Lunar Calculation Functions ====================

int getLunarYearDays(int year) {

int i, sum = 348;

for(i = 0x8000; i > 0x8; i >>= 1) {

sum += (lunarInfo[year - 2000] & i) ? 1 : 0;

}

return sum + getLeapDays(year);

}

int getLeapDays(int year) {

if(getLeapMonth(year)) {

return (lunarInfo[year - 2000] & 0x10000) ? 30 : 29;

}

return 0;

}

int getLeapMonth(int year) {

return lunarInfo[year - 2000] & 0xf;

}

int getLunarMonthDays(int year, int month) {

return (lunarInfo[year - 2000] & (0x10000 >> month)) ? 30 : 29;

}

// Solar to Lunar Conversion

LunarDate solarToLunar(int year, int month, int day) {

LunarDate lunar;

int i, leap = 0, temp = 0;

DateTime base(2000, 1, 31, 0, 0, 0);

DateTime target(year, month, day, 0, 0, 0);

int offset = (target.unixtime() - base.unixtime()) / 86400;

for(i = 2000; i < 2051 && offset > 0; i++) {

temp = getLunarYearDays(i);

offset -= temp;

}

if(offset < 0) {

offset += temp;

i--;

}

lunar.year = i;

leap = getLeapMonth(i);

lunar.isLeap = false;

for(i = 1; i < 13 && offset > 0; i++) {

if(leap > 0 && i == (leap + 1) && lunar.isLeap == false) {

--i;

lunar.isLeap = true;

temp = getLeapDays(lunar.year);

} else {

temp = getLunarMonthDays(lunar.year, i);

}

if(lunar.isLeap == true && i == (leap + 1)) {

lunar.isLeap = false;

}

offset -= temp;

}

if(offset == 0 && leap > 0 && i == leap + 1) {

if(lunar.isLeap) {

lunar.isLeap = false;

} else {

lunar.isLeap = true;

--i;

}

}

if(offset < 0) {

offset += temp;

--i;

}

lunar.month = i;

lunar.day = offset + 1;

return lunar;

}

// ==================== WiFi and NTP Functions ====================

bool connectWiFi() {

Serial.print("Connecting WiFi");

WiFi.begin(ssid, password);

int attempts = 0;

while(WiFi.status() != WL_CONNECTED && attempts < 20) {

delay(500);

Serial.print(".");

attempts++;

}

if(WiFi.status() == WL_CONNECTED) {

Serial.println("\nWiFi Connected!");

Serial.print("IP: ");

Serial.println(WiFi.localIP());

return true;

} else {

Serial.println("\nWiFi Failed!");

return false;

}

}

bool syncTimeWithNTP() {

Serial.println("Syncing NTP...");

configTime(gmtOffset_sec, daylightOffset_sec, ntpServer1, ntpServer2, ntpServer3);

struct tm timeinfo;

int attempts = 0;

while(!getLocalTime(&timeinfo) && attempts < 10) {

Serial.print(".");

delay(1000);

attempts++;

}

if(attempts >= 10) {

Serial.println("\nNTP Sync Failed!");

return false;

}

rtc.adjust(DateTime(timeinfo.tm_year + 1900, timeinfo.tm_mon + 1, timeinfo.tm_mday,

timeinfo.tm_hour, timeinfo.tm_min, timeinfo.tm_sec));

Serial.println("\nNTP Sync Success!");

Serial.printf("Time: %04d-%02d-%02d %02d:%02d:%02d\n",

timeinfo.tm_year + 1900, timeinfo.tm_mon + 1, timeinfo.tm_mday,

timeinfo.tm_hour, timeinfo.tm_min, timeinfo.tm_sec);

WiFi.disconnect(true);

WiFi.mode(WIFI_OFF);

return true;

}

// ==================== Animation Functions ====================

void slideLeftAnimation() {

for(int x = 0; x < SCREEN_WIDTH; x += 8) {

display.clearDisplay();

display.setCursor(-x, 20);

display.setTextSize(2);

display.print("Loading...");

display.display();

delay(10);

}

}

void pageTransition() {

for(int x = 0; x <= SCREEN_WIDTH; x += 16) {

display.clearDisplay();

display.fillRect(0, 0, SCREEN_WIDTH - x, SCREEN_HEIGHT, SSD1306_BLACK);

display.display();

delay(10);

}

}

// ==================== Display Page Functions ====================

// Page 0: Large Digital Clock

void displayPage0(DateTime now) {

display.clearDisplay();

// Date

display.setTextSize(1);

display.setCursor(10, 0);

display.printf("%04d-%02d-%02d", now.year(), now.month(), now.day());

// Weekday

display.setCursor(90, 0);

display.print(weekDays[now.dayOfTheWeek()]);

// Large Time

display.setTextSize(3);

display.setCursor(10, 20);

display.printf("%02d:%02d", now.hour(), now.minute());

// Seconds

display.setTextSize(2);

display.setCursor(100, 28);

display.printf("%02d", now.second());

// Temperature

float temp = rtc.getTemperature();

display.setTextSize(1);

display.setCursor(25, 55);

display.printf("Temp: %.1fC", temp);

display.display();

}

// Page 1: Lunar Calendar Display

void displayPage1(DateTime now) {

display.clearDisplay();

// Solar Date

display.setTextSize(1);

display.setCursor(0, 0);

display.printf("Solar:%04d-%02d-%02d", now.year(), now.month(), now.day());

// Time

display.setTextSize(2);

display.setCursor(20, 15);

display.printf("%02d:%02d:%02d", now.hour(), now.minute(), now.second());

// Lunar Date

LunarDate lunar = solarToLunar(now.year(), now.month(), now.day());

display.setTextSize(1);

display.setCursor(0, 38);

display.printf("Lunar: %d Year", lunar.year);

display.setCursor(0, 50);

if(lunar.isLeap) display.print("Leap ");

display.printf("M%d D%d", lunar.month, lunar.day);

display.display();

}

// Page 2: Detailed Information

void displayPage2(DateTime now) {

display.clearDisplay();

// Title

display.setTextSize(1);

display.setCursor(30, 0);

display.print("System Info");

// Time

display.setCursor(0, 15);

display.printf("Time: %02d:%02d:%02d", now.hour(), now.minute(), now.second());

// Date

display.setCursor(0, 27);

display.printf("Date: %02d/%02d/%04d", now.month(), now.day(), now.year());

// Weekday

display.setCursor(0, 39);

display.print("Week: ");

display.print(weekDays[now.dayOfTheWeek()]);

// Temperature

float temp = rtc.getTemperature();

display.setCursor(0, 51);

display.printf("Temp: %.2f C", temp);

display.display();

}

// Page 3: Analog Clock Face

void displayPage3(DateTime now) {

display.clearDisplay();

int centerX = 64;

int centerY = 36;

int radius = 26;

// Draw clock circle

display.drawCircle(centerX, centerY, radius, SSD1306_WHITE);

display.drawCircle(centerX, centerY, radius - 1, SSD1306_WHITE);

// Draw hour markers

for(int i = 0; i < 12; i++) {

float angle = (i * 30 - 90) * PI / 180;

int x1 = centerX + (radius - 4) * cos(angle);

int y1 = centerY + (radius - 4) * sin(angle);

int x2 = centerX + (radius - 1) * cos(angle);

int y2 = centerY + (radius - 1) * sin(angle);

display.drawLine(x1, y1, x2, y2, SSD1306_WHITE);

}

// Hour hand

float hourAngle = ((now.hour() % 12) * 30 + now.minute() * 0.5 - 90) * PI / 180;

int hx = centerX + (radius - 12) * cos(hourAngle);

int hy = centerY + (radius - 12) * sin(hourAngle);

display.drawLine(centerX, centerY, hx, hy, SSD1306_WHITE);

display.drawLine(centerX + 1, centerY, hx + 1, hy, SSD1306_WHITE);

// Minute hand

float minAngle = (now.minute() * 6 - 90) * PI / 180;

int mx = centerX + (radius - 6) * cos(minAngle);

int my = centerY + (radius - 6) * sin(minAngle);

display.drawLine(centerX, centerY, mx, my, SSD1306_WHITE);

// Second hand

float secAngle = (now.second() * 6 - 90) * PI / 180;

int sx = centerX + (radius - 2) * cos(secAngle);

int sy = centerY + (radius - 2) * sin(secAngle);

display.drawLine(centerX, centerY, sx, sy, SSD1306_WHITE);

// Center dot

display.fillCircle(centerX, centerY, 2, SSD1306_WHITE);

// Digital time at top

display.setTextSize(1);

display.setCursor(35, 0);

display.printf("%02d:%02d:%02d", now.hour(), now.minute(), now.second());

display.display();

}

// ==================== Main Program ====================

void setup() {

Serial.begin(115200);

delay(1000);

Serial.println("\n=== ESP32-C6 Ultimate Clock System ===");

// Initialize I2C

Wire.begin(I2C_SDA, I2C_SCL);

// Initialize OLED

if(!display.begin(SSD1306_SWITCHCAPVCC, OLED_ADDR)) {

Serial.println("OLED Init Failed!");

while(1);

}

Serial.println("OLED Init OK");

display.clearDisplay();

display.setTextColor(SSD1306_WHITE);

display.setTextSize(1);

display.setCursor(15, 10);

display.println("Ultimate Clock");

display.setCursor(20, 25);

display.println("Starting...");

display.display();

delay(1000);

// Initialize DS3231

if(!rtc.begin()) {

Serial.println("DS3231 Init Failed!");

display.clearDisplay();

display.setCursor(10, 20);

display.println("RTC Error!");

display.display();

while(1);

}

Serial.println("DS3231 Init OK");

// NTP Time Sync

display.clearDisplay();

display.setCursor(5, 20);

display.println("Connecting WiFi...");

display.display();

if(connectWiFi()) {

display.clearDisplay();

display.setCursor(15, 20);

display.println("Syncing NTP...");

display.display();

if(syncTimeWithNTP()) {

display.clearDisplay();

display.setCursor(15, 20);

display.println("Time Synced!");

display.display();

delay(1500);

} else {

display.clearDisplay();

display.setCursor(10, 15);

display.println("NTP Sync Failed");

display.setCursor(10, 30);

display.println("Using RTC Time");

display.display();

delay(2000);

}

} else {

display.clearDisplay();

display.setCursor(15, 15);

display.println("WiFi Failed");

display.setCursor(10, 30);

display.println("Using RTC Time");

display.display();

delay(2000);

}

// Startup Animation

slideLeftAnimation();

Serial.println("System Ready!");

lastPageSwitch = millis();

}

void loop() {

DateTime now = rtc.now();

// Auto page switching

if(millis() - lastPageSwitch > PAGE_SWITCH_INTERVAL * 1000) {

pageTransition();

currentPage = (currentPage + 1) % 4;

lastPageSwitch = millis();

}

// Display current page

switch(currentPage) {

case 0:

displayPage0(now);

break;

case 1:

displayPage1(now);

break;

case 2:

displayPage2(now);

break;

case 3:

displayPage3(now);

break;

}

// Serial output

if(now.second() % 10 == 0) {

LunarDate lunar = solarToLunar(now.year(), now.month(), now.day());

Serial.printf("Date: %04d-%02d-%02d %s %02d:%02d:%02d | Lunar: M%d D%d | Temp: %.1fC\n",

now.year(), now.month(), now.day(), weekDays[now.dayOfTheWeek()],

now.hour(), now.minute(), now.second(),

lunar.month, lunar.day, rtc.getTemperature());

delay(1000);

}

delay(100);

}