/*!

\file    main.c

\brief   GD32VW553 Wifi Client + WebServerHtml控制页面 + 多模式LED 控制

\author  阿峰不坏

\company 萤火工场

*/

#include <stdint.h>

#include <stdio.h>

#include <string.h>

#include "FreeRTOS.h"

#include "task.h"

#include "gd32vw55x.h"

#include "app_cfg.h"

#include "gd32vw55x_platform.h"

#include "lwip/sockets.h"

#include "lwip/priv/sockets_priv.h"

#include "wifi_management.h"

#include "wifi_init.h"

#define LED_PORT    GPIOB

#define LED_PIN     GPIO_PIN_2

// 修改原先的 AP_SSID 和 AP_PASSWORD 为 STA 的凭据

#define STA_SSID     "NETGEAR"      // 替换为您的路由器 SSID

#define STA_PASSWORD "xxxxx"  // 替换为您的 Wi-Fi 密码

#define STA_SECURITY AUTH_MODE_WPA_WPA2      // 根据路由器设置调整

#define HTTP_PORT   80

// 中控服务器配置（运行 light_control_server.py 的 IP 和端口）

#define LIGHT_SERVER_IP   "192.168.3.26"

#define LIGHT_SERVER_PORT 8080

//添加全局模式枚举与状态变量

typedef enum {

LED_MODE_OFF,

LED_MODE_ON,

LED_MODE_SLOW_BLINK,    // 慢闪

LED_MODE_FAST_BLINK,    // 快闪

LED_MODE_HEARTBEAT,     // 心跳

LED_MODE_BREATH         // 呼吸

} led_mode_t;

static volatile led_mode_t g_led_mode = LED_MODE_OFF;

//static bool led_physical_state = false; // 记录当前物理电平

void led_init(void) {

printf("LED init\r\n");

rcu_periph_clock_enable(RCU_GPIOB);

gpio_mode_set(LED_PORT, GPIO_MODE_OUTPUT, GPIO_PUPD_NONE, LED_PIN);

gpio_output_options_set(LED_PORT, GPIO_OTYPE_PP, GPIO_OSPEED_MAX, LED_PIN);

gpio_bit_reset(LED_PORT, LED_PIN);

}

// LED 管理任务（独立运行）

static void led_control_task(void *arg) {

printf("LED control task started!\r\n");

int breath_level = 0;

int breath_dir = 1;

while (1) {

switch (g_led_mode) {

case LED_MODE_OFF:

gpio_bit_reset(LED_PORT, LED_PIN);

vTaskDelay(pdMS_TO_TICKS(100));   // 100ms 阻塞

break;

case LED_MODE_ON:

gpio_bit_set(LED_PORT, LED_PIN);

vTaskDelay(pdMS_TO_TICKS(100));

break;

case LED_MODE_SLOW_BLINK:

gpio_bit_set(LED_PORT, LED_PIN);

vTaskDelay(pdMS_TO_TICKS(500));

gpio_bit_reset(LED_PORT, LED_PIN);

vTaskDelay(pdMS_TO_TICKS(500));

break;

case LED_MODE_FAST_BLINK:

gpio_bit_set(LED_PORT, LED_PIN);

vTaskDelay(pdMS_TO_TICKS(100));

gpio_bit_reset(LED_PORT, LED_PIN);

vTaskDelay(pdMS_TO_TICKS(100));

break;

case LED_MODE_HEARTBEAT:

gpio_bit_set(LED_PORT, LED_PIN);

vTaskDelay(pdMS_TO_TICKS(150));

gpio_bit_reset(LED_PORT, LED_PIN);

vTaskDelay(pdMS_TO_TICKS(150));

gpio_bit_set(LED_PORT, LED_PIN);

vTaskDelay(pdMS_TO_TICKS(150));

gpio_bit_reset(LED_PORT, LED_PIN);

vTaskDelay(pdMS_TO_TICKS(800));

break;

case LED_MODE_BREATH:

// 呼吸模式仍使用微秒级延时（总耗时短），但外部循环由 vTaskDelay 控制

for (int i = 0; i < 10; i++) {

gpio_bit_set(LED_PORT, LED_PIN);

sys_us_delay(breath_level * 3 * 1000);

gpio_bit_reset(LED_PORT, LED_PIN);

sys_us_delay((10 - breath_level) * 3 * 1000);

breath_level += breath_dir;

if (breath_level >= 10) { breath_level = 9; breath_dir = -1; }

else if (breath_level < 0) { breath_level = 0; breath_dir = 1; }

}

// 每完成一次呼吸周期，让出 CPU

vTaskDelay(pdMS_TO_TICKS(10));

break;

}

}

}

void led_on(void) { gpio_bit_set(LED_PORT, LED_PIN); }

void led_off(void) { gpio_bit_reset(LED_PORT, LED_PIN); }

// 解析 URL 参数

int get_query_param(const char* request, const char* key, char* value, size_t val_size) {

const char* query_start = strchr(request, '?');

if (!query_start) return 0;

const char* key_pos = strstr(query_start, key);

if (!key_pos) return 0;

const char* eq = strchr(key_pos, '=');

if (!eq) return 0;

eq++;

const char* end = eq;

while (*end && *end != ' ' && *end != '\r' && *end != '&' && *end != '\n') {

end++;

}

size_t len = (end - eq) < (val_size - 1) ? (end - eq) : (val_size - 1);

memcpy(value, eq, len);

value[len] = '\0';

return 1;

}

// 发送 HTTP 响应

void send_response(int fd, const char* content_type, const char* body) {

char header[256];

int len = strlen(body);

snprintf(header, sizeof(header),

"HTTP/1.1 200 OK\r\n"

"Content-Type: %s\r\n"

"Content-Length: %d\r\n"

"Connection: close\r\n"

"\r\n", content_type, len);

write(fd, header, strlen(header));

write(fd, body, len);

}

/* -------------------- 雷士吸顶灯控制（转发至中控服务器） -------------------- */

/**

* @brief 向中控服务器发送 JSON 请求

* @param json_body  JSON 字符串，如 "{\"action\":\"on\"}"

* @return 0 成功，-1 失败

*/

static int send_light_request(const char *json_body) {

int sock = -1;

struct sockaddr_in server_addr;

char request[512];

char response[256];

int ret;

sock = socket(AF_INET, SOCK_STREAM, 0);

if (sock < 0) {

printf("Light socket create failed\r\n");

return -1;

}

memset(&server_addr, 0, sizeof(server_addr));

server_addr.sin_family = AF_INET;

server_addr.sin_port = htons(LIGHT_SERVER_PORT);

if (inet_pton(AF_INET, LIGHT_SERVER_IP, &server_addr.sin_addr) <= 0) {

printf("Invalid light server IP\r\n");

close(sock);

return -1;

}

if (connect(sock, (struct sockaddr *)&server_addr, sizeof(server_addr)) < 0) {

printf("Connect to light server failed\r\n");

close(sock);

return -1;

}

snprintf(request, sizeof(request),

"POST /api/light/control HTTP/1.1\r\n"

"Host: %s:%d\r\n"

"Content-Type: application/json\r\n"

"Content-Length: %d\r\n"

"Connection: close\r\n"

"\r\n"

"%s",

LIGHT_SERVER_IP, LIGHT_SERVER_PORT, (int)strlen(json_body), json_body);

if (write(sock, request, strlen(request)) < 0) {

printf("Write to light server failed\r\n");

close(sock);

return -1;

}

// 读取响应（仅用于判断成功/失败）

ret = read(sock, response, sizeof(response) - 1);

close(sock);

if (ret > 0) {

response[ret] = '\0';

if (strstr(response, "\"status\":\"success\"")) {

return 0;

} else {

printf("Light server error: %s\r\n", response);

return -1;

}

} else {

printf("No response from light server\r\n");

return -1;

}

}

/* 便捷控制函数（供 HTTP API 调用） */

static int light_action_on(void) {

return send_light_request("{\"action\":\"on\"}");

}

static int light_action_off(void) {

return send_light_request("{\"action\":\"off\"}");

}

static int light_action_brightness(int level) {

if (level < 0) level = 0;

if (level > 100) level = 100;

char buf[64];

snprintf(buf, sizeof(buf), "{\"action\":\"brightness\",\"value\":%d}", level);

return send_light_request(buf);

}

static int light_action_scene(int mode) {

char buf[64];

snprintf(buf, sizeof(buf), "{\"action\":\"scene\",\"value\":%d}", mode);

return send_light_request(buf);

}

static void http_server_serve(int conn_fd) {

char buffer[1024];

int len = read(conn_fd, buffer, sizeof(buffer) - 1);

if (len <= 0) {

close(conn_fd);

return;

}

buffer[len] = '\0';

printf("Request: %.100s\r\n", buffer);

// ---- 1. 本地 LED 控制（原功能） ----

if (strncmp(buffer, "GET /api/control", 16) == 0) {

char mode_str[16] = {0};

if (get_query_param(buffer, "mode", mode_str, sizeof(mode_str))) {

if (strcmp(mode_str, "on") == 0) g_led_mode = LED_MODE_ON;

else if (strcmp(mode_str, "off") == 0) g_led_mode = LED_MODE_OFF;

else if (strcmp(mode_str, "slow") == 0) g_led_mode = LED_MODE_SLOW_BLINK;

else if (strcmp(mode_str, "heart") == 0) g_led_mode = LED_MODE_HEARTBEAT;

else if (strcmp(mode_str, "breath") == 0) g_led_mode = LED_MODE_BREATH;

}

const char* mode_names[] = {"off", "on", "slow", "fast", "heart", "breath"};

char json[64];

snprintf(json, sizeof(json), "{\"mode\":\"%s\"}", mode_names[g_led_mode]);

send_response(conn_fd, "application/json", json);

close(conn_fd);

return;

}

// ---- 2. 雷士吸顶灯控制（新增 API，不变） ----

if (strncmp(buffer, "GET /api/light", 14) == 0) {

char action_str[16] = {0};

char value_str[16] = {0};

int result = -1;

char resp_body[128];

if (!get_query_param(buffer, "action", action_str, sizeof(action_str))) {

snprintf(resp_body, sizeof(resp_body), "{\"status\":\"error\",\"msg\":\"missing action\"}");

send_response(conn_fd, "application/json", resp_body);

close(conn_fd);

return;

}

if (strcmp(action_str, "on") == 0) {

result = light_action_on();

} else if (strcmp(action_str, "off") == 0) {

result = light_action_off();

} else if (strcmp(action_str, "brightness") == 0) {

if (get_query_param(buffer, "value", value_str, sizeof(value_str))) {

int level = atoi(value_str);

result = light_action_brightness(level);

} else {

snprintf(resp_body, sizeof(resp_body), "{\"status\":\"error\",\"msg\":\"missing value for brightness\"}");

send_response(conn_fd, "application/json", resp_body);

close(conn_fd);

return;

}

} else if (strcmp(action_str, "scene") == 0) {

if (get_query_param(buffer, "value", value_str, sizeof(value_str))) {

int mode = atoi(value_str);

result = light_action_scene(mode);

} else {

snprintf(resp_body, sizeof(resp_body), "{\"status\":\"error\",\"msg\":\"missing value for scene\"}");

send_response(conn_fd, "application/json", resp_body);

close(conn_fd);

return;

}

} else {

snprintf(resp_body, sizeof(resp_body), "{\"status\":\"error\",\"msg\":\"unknown action\"}");

send_response(conn_fd, "application/json", resp_body);

close(conn_fd);

return;

}

if (result == 0) {

snprintf(resp_body, sizeof(resp_body), "{\"status\":\"success\",\"msg\":\"light command sent\"}");

} else {

snprintf(resp_body, sizeof(resp_body), "{\"status\":\"error\",\"msg\":\"light server unreachable\"}");

}

send_response(conn_fd, "application/json", resp_body);

close(conn_fd);

return;

}

// ---- 3. 返回 HTML 页面（原功能 + 新增吸顶灯控制按钮） ----

const char* html =

"<!DOCTYPE html><html lang=\"zh-CN\"><head><meta charset=\"UTF-8\"><meta name=\"viewport\" content=\"width=device-width, initial-scale=1.0, user-scalable=no\"><title>GD32VW553 多功能控制</title>"

"<style>*{margin:0;padding:0;box-sizing:border-box}body{min-height:100vh;display:flex;justify-content:center;align-items:center;font-family:'PingFang SC','Microsoft YaHei',sans-serif;background:radial-gradient(ellipse at top,#1b2735 0%,#090a0f 100%);padding:20px}.card{width:100%;max-width:420px;padding:30px 25px 35px;background:rgba(255,255,255,0.08);backdrop-filter:blur(12px);-webkit-backdrop-filter:blur(12px);border-radius:32px;border:1px solid rgba(255,255,255,0.15);box-shadow:0 25px 50px rgba(0,0,0,0.6);text-align:center;color:#fff}.title{font-size:24px;font-weight:700;letter-spacing:1px;background:linear-gradient(135deg,#f6d365,#fda085);-webkit-background-clip:text;-webkit-text-fill-color:transparent;background-clip:text}.subtitle{font-size:14px;opacity:0.7;margin-top:4px;letter-spacing:2px}.info-bar{display:flex;justify-content:space-between;font-size:12px;background:rgba(0,0,0,0.3);border-radius:20px;padding:8px 16px;margin:16px 0 20px;opacity:0.8}.info-bar span{display:flex;align-items:center;gap:6px}.led-wrapper{display:flex;flex-direction:column;align-items:center;margin:10px 0 16px}.led{width:120px;height:120px;border-radius:50%;transition:background 0.3s,box-shadow 0.3s;position:relative}.led.off{background:radial-gradient(circle at 35% 30%,#5a5a5a,#1a1a1a);box-shadow:0 0 15px rgba(0,0,0,0.8),inset 0 -6px 10px rgba(0,0,0,0.6);animation:none}.led.on{background:radial-gradient(circle at 35% 30%,#ff7e7e,#e60000);box-shadow:0 0 40px #ff4d4d,0 0 80px rgba(255,50,50,0.5),inset 0 -8px 20px rgba(0,0,0,0.3);animation:none}.led.slow{background:radial-gradient(circle at 35% 30%,#ff7e7e,#e60000);animation:slowBlink 1s step-end infinite}@keyframes slowBlink{0%,100%{box-shadow:0 0 40px #ff4d4d,0 0 80px rgba(255,50,50,0.5),inset 0 -8px 20px rgba(0,0,0,0.3)}50%{box-shadow:0 0 5px #ff4d4d,0 0 20px rgba(255,50,50,0.2),inset 0 -8px 20px rgba(0,0,0,0.6);background:radial-gradient(circle at 35% 30%,#cc5555,#880000)}}.led.heart{background:radial-gradient(circle at 35% 30%,#ff7e7e,#e60000);animation:heartbeat 1.25s ease-in-out infinite}@keyframes heartbeat{0%,100%{box-shadow:0 0 5px rgba(255,50,50,0.2),0 0 20px rgba(255,50,50,0.1);transform:scale(0.95)}12.5%{box-shadow:0 0 40px #ff4d4d,0 0 80px rgba(255,50,50,0.5);transform:scale(1.05)}25%{box-shadow:0 0 5px rgba(255,50,50,0.2),0 0 20px rgba(255,50,50,0.1);transform:scale(0.95)}37.5%{box-shadow:0 0 40px #ff4d4d,0 0 80px rgba(255,50,50,0.5);transform:scale(1.05)}50%{box-shadow:0 0 5px rgba(255,50,50,0.2),0 0 20px rgba(255,50,50,0.1);transform:scale(0.95)}}.led.breath{background:radial-gradient(circle at 35% 30%,#ff7e7e,#e60000);animation:breath 2.5s ease-in-out infinite}@keyframes breath{0%,100%{box-shadow:0 0 5px rgba(255,50,50,0.2),0 0 20px rgba(255,50,50,0.1);transform:scale(0.92)}50%{box-shadow:0 0 50px #ff4d4d,0 0 100px rgba(255,50,50,0.6);transform:scale(1.08)}}.status-text{font-size:20px;font-weight:600;margin-top:12px;letter-spacing:1px}.status-text .on{color:#ff6b6b}.status-text .off{color:#aaa}.btn{display:inline-block;padding:12px 20px;margin:8px 4px;font-size:16px;font-weight:700;border:none;border-radius:50px;color:#fff;background:linear-gradient(135deg,#667eea,#764ba2);box-shadow:0 4px 15px rgba(102,126,234,0.4);cursor:pointer;transition:all 0.25s;min-width:80px}.btn:active{transform:scale(0.94)}.btn.active{background:linear-gradient(135deg,#f093fb,#f5576c);box-shadow:0 0 20px #f5576c}.btn-group{display:flex;flex-wrap:wrap;justify-content:center;gap:8px;margin-top:18px}.footer{margin-top:20px;font-size:13px;opacity:0.5;letter-spacing:0.5px}#modeDisplay{margin-top:10px;font-size:16px;opacity:0.8}hr{border-color:rgba(255,255,255,0.1);margin:20px 0}.section-title{font-size:18px;font-weight:600;opacity:0.8;margin-top:10px}</style></head>"

"<body><div class=\"card\">"

"<div class=\"title\">✨ GD32VW553</div>"

"<div class=\"subtitle\">阿峰不坏 · 萤火工场</div>"

"<div class=\"info-bar\"><span>📶 <span id=\"ssid\">GD32_IOT_AP</span></span><span>🌐 <span id=\"ip\">192.168.3.110</span></span></div>"

// ---- 板载 LED 控制区 ----

"<div class=\"led-wrapper\"><div id=\"led\" class=\"led off\"></div>"

"<div id=\"status\" class=\"status-text\">LED 当前：<span class=\"off\">关闭</span></div></div>"

"<div class=\"btn-group\">"

"<button data-mode=\"on\" class=\"btn\">常亮</button>"

"<button data-mode=\"slow\" class=\"btn\">慢闪</button>"

"<button data-mode=\"heart\" class=\"btn\">❤️ 心跳</button>"

"<button data-mode=\"breath\" class=\"btn\">🌬️ 呼吸</button>"

"<button data-mode=\"off\" class=\"btn\">关闭</button>"

"</div>"

"<div id=\"modeDisplay\">当前模式：关闭</div>"

"<hr>"

// ---- 雷士吸顶灯控制区 ----

"<div class=\"section-title\">💡 雷士吸顶灯</div>"

"<div class=\"btn-group\">"

"<button data-light-action=\"on\" class=\"btn\">开灯</button>"

"<button data-light-action=\"off\" class=\"btn\">关灯</button>"

"</div>"

"<div class=\"btn-group\">"

"<button data-light-action=\"brightness\" data-value=\"25\" class=\"btn\">25%</button>"

"<button data-light-action=\"brightness\" data-value=\"50\" class=\"btn\">50%</button>"

"<button data-light-action=\"brightness\" data-value=\"80\" class=\"btn\">80%</button>"

"<button data-light-action=\"brightness\" data-value=\"100\" class=\"btn\">100%</button>"

"</div>"

"<div class=\"btn-group\">"

"<button data-light-action=\"scene\" data-value=\"7\" class=\"btn\">温馨</button>"

"<button data-light-action=\"scene\" data-value=\"8\" class=\"btn\">影音</button>"

"<button data-light-action=\"scene\" data-value=\"9\" class=\"btn\">阅读</button>"

"<button data-light-action=\"scene\" data-value=\"16\" class=\"btn\">冬暖</button>"

"<button data-light-action=\"scene\" data-value=\"17\" class=\"btn\">夏凉</button>"

"</div>"

"<div class=\"footer\">连接开发板 Wi-Fi 即可控制</div>"

"</div>"   // card 结束

// ---- JavaScript ----

"<script>"

"(function(){"

// 板载 LED 控制（原）

"const led=document.getElementById('led');"

"const statusSpan=document.querySelector('#status .off, #status .on');"

"const modeDisplay=document.getElementById('modeDisplay');"

"const buttons=document.querySelectorAll('.btn[data-mode]');"

"const modeNames={off:'关闭',on:'常亮',slow:'慢闪',fast:'快闪',heart:'心跳',breath:'呼吸'};"

"function updateUI(mode){"

"led.className='led';"

"if(mode==='off'||mode==='on'||mode==='slow'||mode==='heart'||mode==='breath'){led.classList.add(mode);}else{led.classList.add('off');}"

"const modeName=modeNames[mode]||mode;"

"if(mode==='on'){statusSpan.innerHTML='LED 当前：<span class=\"on\">开启</span>';}"

"else if(mode==='off'){statusSpan.innerHTML='LED 当前：<span class=\"off\">关闭</span>';}"

"else{statusSpan.innerHTML='LED 当前：<span class=\"on\">'+modeName+'</span>';}"

"modeDisplay.textContent='当前模式：'+modeName;"

"buttons.forEach(btn=>{btn.classList.remove('active');if(btn.dataset.mode===mode){btn.classList.add('active');}});"

"}"

"function sendMode(mode){"

"fetch('/api/control?mode='+mode).then(res=>res.json()).then(data=>{if(data.mode){updateUI(data.mode);}else{updateUI(mode);}}).catch(err=>{console.error('请求失败:',err);updateUI(mode);});"

"}"

"buttons.forEach(btn=>{btn.addEventListener('click',function(){const mode=this.dataset.mode;sendMode(mode);});});"

"fetch('/api/control?mode=get').then(res=>res.json()).then(data=>{if(data.mode){updateUI(data.mode);}else{updateUI('off');}}).catch(()=>{updateUI('off');});"

// 新增：吸顶灯控制

"function sendLightRequest(action, value){"

"let url='/api/light?action='+action;"

"if(value!==undefined && value!==null && value!=='') url+='&value='+value;"

"fetch(url).then(res=>res.json()).then(data=>{"

"if(data.status==='success'){console.log('Light control success');}"

"else{console.error('Light error:',data.msg);}"

"}).catch(err=>console.error('Fetch error:',err));"

"}"

"document.querySelectorAll('[data-light-action]').forEach(btn=>{"

"btn.addEventListener('click',function(){"

"const action=this.dataset.lightAction;"

"const value=this.dataset.value;"

"sendLightRequest(action, value);"

"});"

"});"

"})();"

"</script>"

"</body></html>";

send_response(conn_fd, "text/html; charset=utf-8", html);

close(conn_fd);

}

static void http_server_task(void *arg) {

int listen_fd = socket(AF_INET, SOCK_STREAM, 0);

if (listen_fd < 0) {

printf("创建 Socket 失败！\r\n");

return;

}

struct sockaddr_in server_addr = {0};

server_addr.sin_family = AF_INET;

server_addr.sin_port = htons(HTTP_PORT);

server_addr.sin_addr.s_addr = htonl(INADDR_ANY);

if (bind(listen_fd, (struct sockaddr *)&server_addr, sizeof(server_addr)) < 0) {

printf("绑定端口失败！\r\n");

close(listen_fd);

return;

}

if (listen(listen_fd, 5) < 0) {

printf("监听失败！\r\n");

close(listen_fd);

return;

}

printf("HTTP 服务器已启动，请连接 Wi-Fi 后访问 http://192.168.3.110\r\n");

while (1) {

struct sockaddr_in client_addr;

socklen_t len = sizeof(client_addr);

int conn_fd = accept(listen_fd, (struct sockaddr *)&client_addr, &len);

if (conn_fd >= 0) {

printf("客户端已连接：%s\r\n", inet_ntoa(client_addr.sin_addr));

http_server_serve(conn_fd);

}

}

}

void print_all_netif_names(void)

{

struct netif *netif;

printf("当前存在的网络接口：\n");

for (netif = netif_list; netif != NULL; netif = netif->next) {

// netif->name 是两个字符的数组，例如 'w', 'l'

// netif->num 是数字编号，通常为 0, 1, ...

printf("  %c%c%d (IP: %s)\n",

netif->name[0],

netif->name[1],

netif->num,

ipaddr_ntoa(&netif->ip_addr));

}

//printf("共 %d 个接口\n", netif_num); // 可选，netif_num 是全局变量

}

static void wifi_sta_task(void *param) {

// 打印当前所有接口

print_all_netif_names();

printf("正在连接 Wi-Fi：%s ...\r\n", STA_SSID);

// 1. 获取 STA 网络接口

struct netif *sta_netif = netif_find("wl1");

if (sta_netif == NULL) {

printf("无法找到 STA 网络接口 (wl1)！\r\n");

sys_task_delete(NULL);

return;

}

// 2. 设置静态 IP 参数（请替换为你的实际网络参数）

uint32_t ip = ipaddr_addr("192.168.3.110");

uint32_t mask = ipaddr_addr("255.255.255.0");

uint32_t gw = ipaddr_addr("192.168.3.1");

// 3. 启用静态 IP 模式（此标志可能用于控制 DHCP 自动启动）

net_if_use_static_ip(true);

// 4. 设置 IP 地址（会覆盖 DHCP 分配的地址）

net_if_set_ip(sta_netif, ip, mask, gw);

// 5. 可选：停止 DHCP 客户端，防止后续 DHCP 覆盖静态 IP

net_dhcp_stop(sta_netif);

// 6. 确保接口 up

net_if_up(sta_netif);

// 7. 将该接口设为默认路由（如果只有一个接口）

net_if_set_default(sta_netif);

// 8. 启动 STA 模式并连接

int ret = wifi_management_connect((char *)STA_SSID, (char *)STA_PASSWORD, 1);

if (ret != 0) {

printf("STA 连接启动失败！错误码：%d\r\n", ret);

// 可选：在此处回退到 AP 模式或无限重试

sys_task_delete(NULL);

return;

}

// 4. 初始化 LED

led_init();

// 5. 创建 LED 控制任务

sys_task_create_dynamic((const uint8_t *)"led_task", 1024, OS_TASK_PRIORITY(2), led_control_task, NULL);

// 6. 启动 HTTP 服务器（阻塞）

http_server_task(NULL);

sys_task_delete(NULL);

}

int main(void) {

platform_init();

uart_config(UART1, 115200, false, false, false);

printf("^_^ Welcome to Soneedu's DIY ~\r\n");

if (wifi_init()) {

printf("Wi-Fi 初始化失败！\r\n");

while (1);

}

sys_task_create_dynamic((const uint8_t *)"sta_task", 4096, OS_TASK_PRIORITY(0), wifi_sta_task, NULL);

sys_os_start();

while (1);

}